CESSNA P 337 H

Pressurized

Skymaster

N777SN

Quick reference training guide

This training manual cannot be used as a substitute for the official pilots operating handbook.

1. Take off  (normal)

Normal flaps for short runway  1/3

Lead with rear engine

37" 2800 RPM (mind over boost) (33"for training)

· 	 Vr: 						75 KIAS

·	 Flaps up 					80 KIAS

·	 Gear up 					89 KIAS

·	 Cruise Climb					115 KIAS 31” 2450 RPM

·	 Fuel flow  					90/90 lbs green/green

2. Take off  (short runway)

·	Flaps 						1/3 Brakes hold

·	Power 						37" 2800 RPM

·	Fuel flow 					90/90 lbs/hr green/green

·	Brakes 						release

·	Attitude						Low tail

·	Vr 						70 KIAS

·	VCs 						80 KIAS (obstacles)

·	Flaps up 					(80 KIAS)

·	Vy 						Gear up (89 KIAS)

·	Cruise Climb					115 KIAS 31“ 33" 2450RPM

3.	Max. Performance Climb

·	Attitude						B A + 12 degrees

·	Speed 						95 KIAS

·	Power						2600 RPM/37"

·	Fuel flow 					90/90lbs/hr green/green

·	Cowl flaps 					open

4. Normal Cruise Climb

·	Attitude					B A +7 degrees

·	Speed 						120 KIAS

·	Power						31" 2450 RPM (SL. up to 15000)

·	Fuel flow 					90 /90 lbs/hr green/green

·	Cowl flaps  					as required

5. Cruise (65%)

·	Power						2400 RPM 29" up to 14.000 ft

·	Mixture 					70/70 lbs peak EGT -/-50 F

·	Cross feed at level flight  - 		Stop AT 50 LBS - Full or empty 								tanks (either)

NOTE

15.000 ft - 18.000 ft					2500/29"

19.000 ft -  20.000 ft				2600/27"

Add 1" for ISA + 10

6. Descent

·	Cabin pressure -			> set  + 500 feet

·	Power					25"/2400 RPM  (min 20")

·	Mixture 				Adjust

·		Anti Ice				as required

7. Normal Visual Circuit

DOWNWIND

·	Speed					140 KIAS

·	Power					25" / 2400 RPM

·	Approach checklist 			completed

ABEAM-Landing Threshold

·		Timing				3 sec/100 ft

·	Flaps					1/3

·	Gear					down

·	Speed					110 KIAS

·	Power					25" / 2400 RPM

BASE LEG

·	Power					20" / 2400 RPM

·	Flaps					2/3

·	Speed					100 KIAS

·	Before landing checklist		completed

FINAL

·	Power					17"/2400 RPM

·	Speed					90 KIAS

·	Flaps					Full

SHORT FINAL (VTgT Headwind component 10 KTS or less)

·	Speed					85 KIAS

·		Power					as required

·	Propeller levers			max. RPM (below 15")

·	Mixture 				Fully rich

8. Short Field Landing

·	Fly normal pattern

·		Final					Steeper angle

·	Flaps					Full

·	Power					as required / max. RPM (fully fine)

·	Speed					78 KIAS

9. Flapless

DOWNWIND

·	Speed					140 KIAS

·	Power					25" / 2400 RPM

ABEAM LANDING THRESHOLD

·		Timing				3 sec./100ft

·	Gear					Down

·	Speed					135 KIAS

·	Power					25" / 2400 RPM

BASE LEG

·	Power					17"/2400 RPM

·		Speed					105 KIAS

·	Before landing checklist 		completed

FINAL

·	Speed					90 KIAS

·	Power					2400 RPM /as required

·		Short Final				MAX. RPM (below 15")

10. GO AROUND

·	Power					37" / 2800 RPM (33" for training)

·	Flaps					Retract to 1/3

·	Speed					80 KIAS

·	Above obstacles			Flaps up

·	Speed 89 KIAS			Gear up

·	Cowl flaps				Open

Note: Do not retract gear if another approach is to be conducted.

11. Go - no go decision - one engine out

NO GO

·	Airspeed below			80 KIAS

·	Below obstacles			(no possibility for circumnavigation)

·	Runway available			Land

GO

·	Airspeed				> 80 KIAS

·	No runway				Available

·	Above obstacles			above obstacles or possible for circum 							navigation

ENGINE FAILURE AFTER TAKE-OFF ACTIONS

·	Throttles, Props, Mixtures		Full forward

·	Inoperative engine			Identify

·	Inoperative engine			Throttle closed

·	Inoperative engine			Feather

·	Inoperative engine			Cowl flaps close

·	Airspeed 80 KIAS			Flaps up

·	Above obstacles			Accelerate 89 KIAS

·	89 KIAS (bleu line)			Retract gear

·	ATC					Call

Inoperative Engine securing Checklist

·	Check					Box items

·	Mixture				Idle cut off

·	Alternator				Off

·	Ignition Switch			Off

·	Fuel selector				Off (red)

·	Synchrophaser			Off

·	Electrical Load			Reduce

·	Inop. Engine				Attempt restart

ZERO TRUST 			12 MAP

12. Engine failure above 1000ft AGL and in cruise

·		Throttles, Props, Mixtures		full forward (slowly)

·		Inoperative engine			identify

·		Mixture inoperative engine	RICH

·		Aux. Fuel Pump (inop. engine)	On HIGH

·		Fuel selector (inop. engine)	cross feed

·		Check immediately	Restart	On fuel flow

IF NOT

·		Aux. Fuel Pump			OFF

·		Inoperative Engine			Feather /secure

·		Live Engine				Power as required

13. Engine restart in flight.

(Feathered and equipped with unfeather accumulators)

·		Fuel selector 			Green on proper tank

·		Throttle 				Just out of gear warning

·		Propeller				Cruise rpm position

·		UN feathering				Check

·		Aux. fuel pump 			High

·		Ignition				Both or start if no unfeathering

·		Mixture				Adjust for fuel flow 10-35 lbs

·		Aux. fuel pump 			off

·		Oil pressure				Checked

·		Alternator				On and checked

·		Power	 (200 degrees F.)		Reset

14. One engine inoperative Visual approach and landing

DOWNWIND

·	Speed					110 KIAS

·	Power					33 / max. RPM

·	Fuel selector operating engine	Green

·	Approach checklist			completed

BASE LEG

·	On profile				gear down

·	Speed					110 KIAS

·	Power					30" / max. RPM

·	Before landing checklist		completed

FINAL

·	Flaps					1/3

·	Speed					110 KIAS

·	Power					Max. RPM

LANDING ASSURED

·	Flaps					as required

·	Speed 				90 KIAS

·		Threshold				85 KIAS

15. Two Engine Instrument Approach

OUTBOUND

·	Flaps					1/3

·	Gear					Down

·	Speed					120 KIAS

Power					25" 2450 RPM

ON GLIDE (within 10 degrees)

·		Flaps					2/3

·		Speed					110 KIAS

·		Power					20" / 2450 RPM

VISUAL

·		Flaps					As required

·		Speed					90 KIAS

·		Speed on short final			85 KIAS

16. One engine inoperative instrument approach

OUTBOUND

·	Speed					110 KIAS

·	Power					33" / Max. RPM

·	Fuel selector Operating Engine	Green

ON GLIDE

·	Gear					Down

·	Flaps					1/3

·	Speed					110 KIAS

·	Power					30" MAX. RPM

LANDING ASSURED

·	Flaps					As required

·	Speed					90 KIAS

·	Threshold					85 KIAS

Note

Non-precision Approach: No flaps until visual and on glide!

17. Air work

STEEP TURNS 45 degrees

1.	PREPARATION

·	Speed 140 KIAS (2400/ 25")

·	Set heading bug

2.	EXECUTION

·	The exercise consists of two 360 deg. turns to the one direction and immediately followed by two 360 deg. turns opposite.

·	Anticipate heading on roll out +/- 10 deg.

·	Add +/- 2" after 30 deg. bank and reduce again on roll out.

STALLS (pressurization off)

1.	PREPARATION

·	Speed 140 KIAS (2400/ 25")

·	Altitude +2500 ft AGL

·	Area not over populated zones

·	Clearing turns for look out

2.	STALL CLEAN

·	Level flight, set heading bug

·	Power smoothly idle

·	Keep level flight

·	Trim speed 90 KIAS

Recovery: 	Power 33"

Attitude 0 deg.

At 90 KIAS rotate  +10 deg.

Reduce power 31"

3.	STALL GEAR DOWN FLAPS 2/3 30 deg. BANK

·	Level flight set heading bug

·	Flaps 1/3 gear down 110 KIAS

·	Power 2400/20" flaps 2/3

·	Bank 30 deg

·	Trim speed 80 KIAS

Recovery: 	Power 33"

Attitude -5 deg. BA

At 80 KIAS wings level

Flaps 1/3

Rotate + 10 deg.

Positive climb flaps up

89 KIAS gear up

Reduce power 31"

4.	STALL IN LANDING CONFIGURATION

·	Level flight, set heading bug

·	Flaps 1/3 gear down 110 KIAS

·	Power 20"/2400

·	Flaps 2/3 100 KIAS

·	Power 17" /2400

·	Flaps full

·	Trim speed 80 KIAS

Recovery: 	Power 33"

Attitude -5 deg. BA

Flaps 1/3

At 80 rotate + 10 deg.

Positive climb flaps up

89 KIAS gear up

Reduce power 31"

Informatie bijgewerkt tot  31 mei 2000