BEFORE TAKE-OFF

Fuel Selectors	BOTH GREEN

Flaps	0 - 1/3

Cowl Flaps	OPEN

Trims	TAKE-OFF POSITION

Engine Instruments	CHECKED

Synchrophaser	OFF

DESCENT

Cabin Pressure	SET

Mixtures	ADJUST

Power	(MIN 20")

Anti - Ice	AS REQUIRED

Cowl Flaps	CLOSED

1

APPROACH

Altimeters	SET

Fuel	GREEN /GREEN

Aux. Fuel Pumps	OFF

Mixtures	RICH

RPM	2500

Landing - Taxi Lights	ON

Synchrophaser	OFF

2

CLEARED FOR TAKE-OFF

Heater	OFF

Altimeters	(elev.) CHECKED

Pitot Heater	ON

De-Ice /WX Radar	(IF REQUIRED) ON

Landing-Taxi Lights & Strobe Lights	ON

Transponder	 ON

Gyro's (QFU)	CHECKED & SET

BEFORE LANDING

Heater	OFF

Mixtures	RICH

RPM	2500

Gear	DOWN / GREEN

TAKE-OFF

NORMAL	SHORT FIELD

Vr	75 KTS	70 KTS

Vx Se	80 KTS	80 KTS

Vy Se	90 KTS	90 KTS

3

AFTER LANDING

Flaps	RETRACTED

Anti Ice	OFF

Strobe Lights	OFF

Landing & Taxi Lights	AS REQUIRED

Cowl Flaps	OPEN

Transponder & Radar	OFF

4

AFTER TAKE-OFF

Flaps / Gear	UP

Power	33"- 2450 RPM SET

Fuel Flow	GREEN or AS REQUIRED

Cowl Flaps	OPEN AS REQUIRED

Landing & Taxi lights	OFF

Pressurization	CHECKED

Anti-Ice & Heater	AS REQUIRED

5

SHUT DOWN

Turbo Cooling 	5 MIN.

Parking Brake 	SET

Avionics Power Switch & Electrical Equipment	OFF

Mixtures	IDLE CUT OFF

Cowl Flaps	CLOSED

Magneto's Master and Alternator Switches	OFF

Control Lock	INSTALLED

6

CRUISE

Power	SET

Fuel Flow	70lbs / 70/lbs

Synchrophaser	AS DESIRED

Cross feed,	(if required)	MAX 50 LBS

Anti / De-ice	AS REQUIRED

7

Checklist Cessna - P337H - N777SN - version 1.6

HOT / FAILED STARTING

Mixture	IDLE CUT OFF

Throttle	OPEN

Fuel Pump	OFF

Starter	ON A FEW SEC

BEFORE START CHECKLIST

External Inspection	COMPLETED

Gear Selector	DOWN

Papers	ON BOARD

Fuel Selectors	BOTH GREEN

Door & Seat Belts	CLOSED - LOCKED & ON

Parking Brake	APPLIED

Avionics Power Switch	OFF

Fuses	All IN

Magneto Switches	BOTH OFF

All Electrical Switches	OFF

Pressurization Selector	SET & ON

Auto Pilot	OFF

Prop. Sync.	OFF

Auxiliary Air & Heater	CHECKED

Mixtures	IDLE CUT OFF

Radar	OFF

Dump Valves	CLOSED

Primers	LOCKED

Alti Meters & Instruments	CHECKED AND SET

Controls and Trims	CHECKED AND SET

Master Switch	ON - CHECK VOLTAGE

Fuel Quantity	CHECKED GOOD FOR .... HOURS

Alternator switches	< no ground power start! >	 BOTH

Cowl Flaps	OPEN

8

AFTER STARTING

Avionics Power Switch	ON

Ammeter & Voltmeter	CHECKED

Suction systems	CHECKED

Comm / Nav. (GPS)	ON & SET

Gyro's	ON / SET & SLAVED

Flaps	CHECKED & SET

Electrical Trim	CHECKED & SET

Radar	STAND BY

TAXI

Fuel Selectors	BOTH GREEN

Brakes	CHECKED

Flight Instruments	CHECKED

Gyro's and Turn & Bank	CHECKED & SET

RUN UP

Engine Instruments	CHECKED

Mixtures	RICH

Magneto's	(1800 RPM)	 CHECKED

Drop & Difference	MAX 175 RPM & 50 RPM

Magneto Switches	BOTH ALL ENGINES

Propellers (1800)	FEATHERED & SET HIGH

Throttles Idle	1000 RPM

Alternators	CHECKED

Engine Instruments	CHECKED

Trims	TAKE OFF POSITION

STARTING

Anti-collision Light	ON

Throttle ½ Course	OPEN

Propeller	HIGH RPM

Mixture	RICH

Fuel Pump	HIGH

Fuel Flow	CHECKED

Mixture	IDLE CUT OFF

Fuel Pumps	OFF

Throttle	OPEN 1 CM

Starter	ENGAGE

Mixture	RICH

Oil Pressure	CHECKED

The aircraft C337 N777SN is owned by;

Gundel Transport Company

1050 S State Street

Dover, Delaware 19901

USA