| B2C2 Aviation Cessna T337D N2TG STANDARD PROCEDURES PREFLIGHT Hobbs - RECORD Documents – CHECK AROW Control Lock – REMOVE Magnetos – OFF Avionics Master – OFF Circuit Breakers – IN Front Engine Fuel Selector – LEFT MAIN Rear Engine Fuel Selector – RIGHT MAIN Master – ON Fuel Quantity Indicators – CHECK, PUSH AUX Turn and Bank – AUDIBLE Flaps – DOWN, CHECK AILERON DROOP Lights, Stall Horn, and Pitot Heat – CHECK Lights and Pitot Heat - OFF Master – OFF Check Oxygen Pressure – RECORD PRESSURE Check Oxygen Masks Present – OK Right Main Tire – OK Right Main Brakes – OK Right Wing Fuel Sump – OK Baggage Door – SECURE Rear Engine Cowl Covering – REMOVE Rear Engine Oil – min7/max8 Quarts – CHECK, RECORD LEVEL & ADDED Rear Engine Fuel Bleed - CHECK Rear Engine Oil Door – SECURE Rear Engine Exhaust – SECURE Rear Prop – OK, NO LEAKS Rear Spinner - OK Elevator – FREE, CORRECT Beacon, Navlights Antennas – OK Left Wing Flap – OK Left Wing Aileron – FREE, CORRECT Left Wing Tip & Fuel Vent – OK Left Wing Leading Edge – OK Left Wing Tie Down – REMOVE Pitot Tube Cover - REMOVE Left Main Tire – OK Left Brakes - OK Left Wing Fuel Sump – CHECK . | Front Engine Oil – min7/max8 Quarts – CHECK, RECORD LEVEL & ADDED Front Engine Fuel Bleed – CHECK Front Engine Oil Door – SECURE Nose Wheel Gear and Tire – OK Front Engine Cowl Cover - REMOVE Front Prop – OK, NO LEAKS Front Spinner – OK Front Exhaust - OK Right Wing Leading Edge – OK Right Wing Tie Down – REMOVE Right Wing Tip & Fuel Vent – OK Right Wing Aileron – FREE, CORRECT Right Wing Flap – OK STARTING, COLD START Preflight – COMPLETE Brakes - RELEASE Seat Belts and Harnesses – SECURE Master – ON Avionics Master - OFF Regulator – SET 1 OR 2 Landing Gear Handle Neutral – OK Landing Gear Green Light – ON Landing Gear Lights and Horn – PUSH TO TEST Cowl Flaps – OPEN Front Engine Fuel Selector – LEFT MAIN Rear Engine Fuel Selector – RIGHT MAIN Heater Switch – OFF Beacon – ON Mixture – RICH Propeller – HIGH RPM Throttle – CRACKED ONE INCH Front Fuel Pump – HIGH, 3-5 SECONDS →OFF Front Engine Start – 2-3 REVOLUTIONS Alternators - ON Rear Fuel Pump – HIGH, 3-5 SECONDS →OFF Rear Engine Start – 2-3 REVOLUTIONS Mixture - LEAN Oil Pressure – CHECK IN GREEN ≤ 30 SECONDS Engine Start Time - RECORD Flaps – UP Avionics Master – ON JPI 750 – SET FUEL/RECORD Transponder – SQUAWK ALT ATIS – COLLECT Ground Control – CALL TAXI Speed – BRISK WALK Brakes – CHECK Turn Instruments – CHECK | VSI – 70 MPH VY – 117 MPH VSO – 55 MPH VYSE – 100 MPH VA – 155 MPH VX – 86 MP VNE – 225 MPH VXSE – 96 MPH VMAX 1/3 FLAPS – 160 MPH VMAX FULL FLAPS – 108 MPH NORMAL CLIMB 115 MPH VMAX GEAR – 140 MPH CRUISE CLIMB 120-140 MPH BEST GLIDE@4500LB – 116 MPH BEST GLIDE@3700LB - 106 MPH RUN-UP Brakes – SET Cowl Flaps- OPEN Flight Controls – FREE, CORRECT Elevator and Rudder Trim – TAKE OFF SETTINGS Throttles – 1800 RPM Props – FEATHER, CHECK RPM DROP Magnetos – CHECK 50 RPM MAX DIFFERENCE Alternators – CHECK POSITIVE AMPS L,R,BAT Engine Instruments Check – OK Suction Gage Check – OK Throttle 1000 RPM Check Idle – OK Flight Instruments Set – OK AP Check - OK Radios Set, IFR Clearance – OK Autopilot - OFF Cowl Flaps – OPEN Flaps – SET FOR DESIRED TAKEOFF, 1/3 MIN Strobes – ON Navigation Lights - ON Landing Lights – ON Doors and Windows – CLOSED Transponder – SQUAWK ALT Wing Flaps - SET NORMAL TAKEOFF Wing Flaps – 1/3 DOWN Power – FULL THROTTLE, 2800 RPM Engine- MONITOR Elevator – LIFT NOSE WHEEL AT 75 MPH Climb Speed - 115 MPH Brakes – MOMENTARY Landing Gear – RETRACT 500 FT AGL Flaps – RETRACT AFTER OBSTACLES CLEARED Livermore Radio Frequencies ATIS – 119.65 GROUND 121.6 TOWER – 118.1/128.2 EMERGENCY 121.5 NORCAL 123.85(E)/125.35(W) | STANDARD PROCEDURES NORMAL ROBERTSON TAKEOFF Wing Flaps – 2/3 DOWN Power – FULL THROTTLE 2800 RPM Engine – MONITOR Elevator – LIFT NOSE WHEEL AT 60 MPH Air Speed - 75 MPH over 50’ OBSTACLE Air Speed – Nose Down and 100 MPH After 50’ Obstacle Brakes – MOMENTARY Wing Flaps – 1/3 AFTER 100 MPH Landing Gear – RETRACT AFTER 100MPH Climb Speed – 115 MPH Wing Flaps – RETRACT FULL STOL ROBERTSON TAKEOFF Wing Flaps – 2/3 DOWN Brakes - SET Power – FULL THROTTLE 2800 RPM Brakes - RELEASE Engine – MONITOR Elevator – LIFT NOSE WHEEL AT 50 MPH Air Speed - 65 MPH over 50’ OBSTACLE Air Speed – Nose Down and 100 MPH After 50’ Obstacle Brakes – MOMENTARY Wing Flaps – 1/3 AFTER 100 MPH Landing Gear – RETRACT AFTER 100MPH Climb Speed – 115 MPH Wing Flaps – RETRACT FULL FOR GUSTY OR SEVERE CROSSWIND CONDITIONS USE 60MPH FOR NOSE LIFTOFF NORMAL CLIMB Airspeed – 120 TO 140 MPH Power – 28 INCHES and 2600 RPM Mixtures – LEAN TO 14.5 GPH Cowl Flaps – OPEN AS REQUIRED MAXIMUM PERFORMANCE CLIMB SEA LEVEL TO 20,000 FT Airspeed – 117 MPH Power – FULL THROTTLE and 2800 RPM Mixtures – ADJUST TO 21 GPH Cowl Flaps – OPEN AS REQUIRED Check Placard for Power and GPH ≥ 20000 FT |
| --- | --- | --- | --- |
| B2C2 Aviation Cessna T337D N2TG STANDARD PROCEDURES CRUISE POWER SETTINGS Power – 15 TO 28 INCHES, 2200-2600 RPM. SELECT COMBINATION FOR NO MORE THAN 75% POWER Cowl Flaps – CLOSED Engine Temps - MONITOR Elevator and Rudder Trim – ADJUST Mixture – LEAN FOR CRUISE FLOW SETTING Fuel Tanks – MANAGE FUEL. MAIN TANKS - 46 GAL EACH L/R AUX TANKS – 18 GAL EACH L/R TOTAL FUEL 128 GAL/64 GAL EACH L/R DESCENT Power – AS DESIRED Cowl Flaps – CLOSED Engine Temps – MONITOR Wing Flaps – 1/3 DOWN BELOW 160 MPH IN RANGE Autopilot – OFF Mixtures – RICH Front Engine Fuel Selector – LEFT MAIN Rear Engine Fuel Selector – RIGHT MAIN Landing Gear – DOWN BELOW 140 MPH Landing and Taxi Lights - ON BEFORE LANDING Front Engine Fuel Selector – LEFT MAIN Rear Engine Fuel Selector – RIGHT MAIN Mixtures – RICH Propellers – HIGH RPM Cowl Flaps - OPEN Landing Gear – DOWN BELOW 140 MPH Landing Gear Light – GREEN, CHECK GEAR AND DOORS IN MIRRORS Landing Gear Handle – RETURNED TO NEUTRAL Wing Flaps – 2/3 DOWN BELOW 100 MPH Wing Flaps – FULL BELOW 85 MPH Final Approach – 80 MPH Speed at 50’ Obstacle – 75 MPH LANDING GEAR CHECK ON FINAL | VSI – 70 MPH VY – 117 MPH VSO – 55 MPH VYSE – 100 MPH VA – 155 MPH VX – 86 MP VNE – 225 MPH VXSE – 96 MPH VMAX 1/3 FLAPS – 160 MPH VMAX FULL FLAPS – 108 MPH VMAX GEAR – 140 MPH NORMAL CLIMB 115 MPH CRUISE CLIMB 120-140 MPH BEST GLIDE@4500LB – 116 MPH BEST GLIDE@3700LB - 106 MPH NORMAL LANDING Touchdown - 60 MPH Landing Roll – LOWER NOSE GRADUALLY Brakes – MINIMUM REQUIRED ROBERTSON STOL MAXIMUM PERFORMANCE LANDING Final Approach 62 MPH Speed at 50’ Obstacle 62 MPH Touchdown - 50 MPH Flare to Touchdown – USE ELEVATOR AND POWER TO ARREST RATE OF SINK Flaps – IMMEDITELY UP AFTER LANDING Brakes – APPLY HEAVY BRAKING LANDING GEAR CHECK ON FINAL AFTER LANDING Wing Flaps – RETRACT Cowl Flaps – OPEN Landing Lights – OFF SECURING JPI Fuel Remaining - RECORD Parking Brake – SET Magnetos - CHECK Avionics Master – OFF Mixture – IDLE CUTOFF Lights – OFF Magnetos – OFF Alternator – OFF Master – OFF Engine Stop Time - RECORD Control Lock – INSTALL Tie Downs – INSTALL Cowl Covers – INSTALL Pitot Tube Cover – INSTALL CLOSE YOUR FLIGHT PLAN | ENGINE CRUISE PERFORMANCE, 10000FT %BHP vs MP and RPM (ROP) TAS, %BHP vs FF (LOP) TAS vs MP and RPM (ROP) TAS CHANGE=10 MPH, 8.7 Kts PER 5000FT ALT CHANGE | STANDARD PROCEDURES STARTING, HOT START Complete Cold Start Checklist to point of Engine Start, Then: Mixture – RICH Propeller – HIGH RPM Throttle – CRACKED ONE INCH Front Engine Start – 2-3 REVOLUTIONS If NO Start, Front Fuel Pump – HIGH, 3-5 SECONDS →OFF, Repeat Start RearEngine Start – 2-3 REVOLUTIONS If NO Start, Rear Fuel Pump – HIGH, 3-5 SECONDS →OFF, Repeat Start Return to Cold Start Checklist, Complete VOR Guidance Tune VOR frequency. SET NAV/GPS to NAV . Turn OBS to desired radial Set NAV radio OBS to show radial on MX20 AUTOPILOT ILS APPROACH PROCEDURE, VECTORS TO FINAL 5As Tune Localizer Frequency. SET NAV/GPS to NAV Set Autopilot to Heading Mode Set Heading Bug to Vectored Headings Select ALT mode. Press and hold HDG, then press NAV GS mode indicator on once 50% CDI achieved. Once HDG mode indicator goes out, IMMEDIATELY turn Heading Bug to inbound ILS Course Autopilot should capture glide slope and track down the ILS. Press and Hold ALT if GS not Lit. AUTOPILOT ILS APPROACH PROCEDURE, STRAIGHT IN APPROACH 5As Tune Localizer Frequency. SET NAV/GPS to NAV With Aircraft established on ~45 degree intercept course to ILS and Needle deflection less than 75% FS Press NAV to engage ILS Navigation Set Heading Bug to Inbound ILS course Select ALT mode GS mode indicator on once 50% CDI achieved. Autopilot should capture glide slope and track down the ILS. Press and hold ALT if GS not Lit. |
| B2C2 Aviation Cessna T337D N2TG EMERGENCY PROCEDURES LANDING GEAR SYSTEM FAILURE Cycle Gear Handle Several Times Before Starting Emergency Procedure, This May Clear the Problem Landing Gear Handle – FULL DOWN Pump Handle – EXTEND Handle – PUMP UNTIL GREEN LIGHT ON Handle – CONTINUE UNTIL HANDLE RETURNS TO NEUTRAL. ~85 STROKES REQUIRED. RED VOLTS HIGH INDICATION Alternators – OFF Turn Each Alternator OFF then ON IF RED LIGHT TURNS OFF WITH ALTERNATOR OFF, LEAVE ALTERNATOR OFF IF RED LIGHT REMAINS ON, SWITCH TO OTHER REGULATOR IF RED LIGHT REMAINS ON TURN BOTH ALTERNATORS OFF, REDUCE LOAD, LAND ASAP ENGINE OUT ON TAKEOFF, SUFFICENT RUNWAY REMAINING TO LAND OR STOP Cut Power and Decelerate to a Stop ENGINE OUT ON TAKEOFF, INSUFFICENT RUNWAY REMAINING TO LAND OR STOP Mixtures – FULL FORWARD Props – FULL FORWARD Throttles – FULL FORWARD Dead Engine – IDENTIFY RPM Dead Engine – VERIFY THROTTLE Dead Engine Prop – FEATHER Air Speed – MAINTAIN 100 MPH Wing Flaps – RETRACT FULLY IN INCREMENTS Landing Gear – RETRACT Cowl Flaps – OPERATIVE ENGINE FULL OPEN, INOP ENGINE FULL CLOSE | ENGINE OUT DURING FLIGHT Mixtures – FULL FORWARD Props – FULL FORWARD Throttles – FULL FORWARD Dead Engine – IDENTIFY WITH RPM GAGES Dead Engine – VERIFY WITH THROTTLE GO TO INOP ENGINE RESTART CHECKLIST >> Dead Engine Prop – FEATHER Cowl Flaps – OPERATIVE ENGINE FULL OPEN, INOP ENGINE FULL CLOSE Mixture – OPERATIVE ENGINE ADJUST AS NECESSARY GO TO INOPERATIVE ENGINE SECURE >> INOPERATIVE ENGINE RESTART Fuel Flow – CHECK Aux Fuel Pump – ON Fuel Selector – SWITCH TANKS Fuel Pump – HIGH Magnetos – CHECK BOTH IF UNSUCCESSFUL COMPLETE ENGINE OUT>> INOPERATIVE ENGINE SECURE Magnetos – OFF Alternator – OFF Mixture – IDLE CUTOFF Aux Fuel Pump – OFF Cowl Flaps – CLOSE INOP ENGINE Fuel Selector – OFF ENGINE RESTART AFTER FEATHERING Avionics Master - OFF Front Engine Fuel Selector – LEFT MAIN Rear Engine Fuel Selector – RIGHT MAIN Throttle – ADVANCE ONE INCH Propeller – CRUISE RPM Air Speed – GREATER THAN 115 MPH Aux Fuel Pump Switch – HIGH Mixture – ADJUST FOR 2-6 GPH Magnetos – START UNTIL WINDMILLING Mixture – ADJUST FF FOR MAX ACCELERATION TO 1000 RPM Aux Fuel Pump Switch – OFF Alternator - ON Throttle – ADJUST FOR SMOOTH ACCELERATION Power – INCREASE SLOWLY UNTIL CHT >200F Avionics Master – ON Transponder – SQUAWK ALT | EMERGENCY PROCEDURES SINGLE ENGINE APPROACH Landing Gear – EXTEND ON DOWNWIND Wing Flaps – MINIMUM UNTIL LANDING ASSURED Airspeed – 90-100 MPH ON APPROACH SINGLE ENGINE GO AROUND Throttle – FULL FORWARD 2800 RPM Airspeed – 100 MPH Wing Flaps – RETRACT TO 1/3 Cowl Flaps – OPEN ON OPERATING ENGINE Wing Flaps – RETRACT AFTER OBSTACLES CLEARED Landing Gear – RETRACT AFTER OBSTACLES CLEARED FORCED LANDING – PRECAUTIONARY LANDING WITH POWER Drag Over Selected Field with Flat and 100 MPH Airspeed. Smooth Hard Surface – WHEELS DOWN LANDING FULL FLAPS HOLD NOSE WHELL OFF AS LONG AS POSSIBLE Rough or Soft Surface – WHEELS UP LANDING FLAPS DOWN APPROACH 90 MPH ALL SWITCHES OFF EXCEPT MAGNETOS CABIN DOOR UNLATCH PRIOR TO FLARE POWER TO MINIMUM ON FLARE MAGNETOS OFF PRIOR TO CONTACT LANDING SLIGHTLY TAIL LOW ATTEMPT HOLD TAIL LOW DURING SLIDE EMERGENCY DESCENT FROM ALTITUDE Throttles - IDLE Flaps – UP Landing Gear- UP Nose Down, Spiral Down Airspeed – 190 MPH MAX = STRUCTURAL CRUISING SPEED | FORCED LANDING - COMPLETE POWER LOSS FEATHER AND SECURE BOTH ENGINES ALL SWITCHES OFF EXCEPT MASTER Smooth Hard Surface – GEAR DOWN LANDING APPROACH AT 96 MPH EXTEND GEAR MANUALLY- MANUAL EXTENSION REQUIRES 85 STROKES OF HANDLE FLAPS AS NECESSARY TURN OFF MASTER HOLD NOSE WHELL OFF AS LONG AS POSSIBLE Rough or Soft Surface – GEAR UP LANDING APPROACH AT 96 MPH FLAPS DOWN IF PRACTICAL WITHIN GLIDING DISTANCE OF THE FIELD CABIN DOOR UNLATCH PRIOR TO FLARE POWER TO MINIMUM ON FLARE TURN OFF MASTER LANDING SLIGHTLY TAIL LOW ATTEMPT HOLD TAIL LOW DURING SLIDE LANDING WITHOUT POSITIVE INDICATION OF GEAR LOCKING Gear is observed to be down, but gear light is out or flickers Make a Normal Full Flaps Approach Landing Gear Handle – HOLD DOWN DURING LANDING Engine RPM – MAINTAIN 1000 RPM MINIMUM BOTH ENGINES Taxi – CLEAR OF RUNWAY BEFORE RELEASING GEAR HANDLE OR REDUCING FRONT ENGINE RPM Rear Engine – SHUTDOWN Tail Section – DEPRESS TO LIFT NOSE WHEEL OFF GROUND Front Engine – SHUTDOWN Nose Gear – CHECK LOCKED BEFORE LOWERING TO GROUND |

| B2C2 Aviation Cessna T337D N2TG EMERGENCY PROCEDURES NOSE GEAR VERIFIED NOT DOWN OR PARTIALLY DOWN, MAIN GEAR DOWN Transfer Gear and Passengers as far back as possible Select a Hard Surface or Smooth Sod Runway Landing Gear Handle – DOWN Flaps – FULL DOWN Master – OFF Door – OPEN TO PREVENT JAMMING Landing - SLIGHTLY TAIL LOW Mixtures – IDLE CUT OFF Magnetos – OFF Nose Wheel – HOLD OFF AS LONG AS POSSIBLE Fuel Selectors – OFF EXIT THE AIRCRAFT ASAP MAIN GEAR VERIFIED NOT DOWN OR PARTIALLY DOWN Landing Will be Wheels Up Landing Gear – RETRACT Select a Hard Surface or Smooth Sod Runway Front Engine SHUTDOWN AND SECURE Front Prop – POSITION HORIZONTALLY WITH STARTER Airspeed – 90 MPH ON APPROACH Flaps – FULL DOWN ALL SWITCHES OFF EXCEPT REAR ENGINE MAGNETOS. Door – OPEN TO PREVENT JAMMING Landing - LEVEL ATTITUDE Mixtures – IDLE CUT OFF Magnetos – OFF Fuel Selectors – OFF EXIT THE AIRCRAFT ASAP | ELECTRICAL FIRE IN FLIGHT Master – OFF Avionics – OFF All Electrical - OFF Oxygen – OFF Cabin Air, Heat Vents – CLOSE Fire – EXTINGUISH Breathing Hood – USE IF NECESSARY Once Fire is Out Breakers – CHECK, DO NOT RESET Avionics – EACH RADIO OFF Master – ON Avionics Master – ON Electrical Switches – ON ONE AT A TIME Cabin Air, Heat, Vents – OPEN ONCE FIRE IS COMPLETELY OUT ENGINE FIRE IN FLIGHT Fuel Selector For Engine On Fire – OFF Mixture for Engine On Fire – IDLE CUT OFF Aux Fuel Pump For Engine On Fire – OFF Magnetos For Engine on Fire – OFF DECLARE EMERGENCY Master – OFF Cabin Heat and Air – OFF LAND IMMEDIATELY SUSPECTED HEATER FIRE Follow Front Engine Securing Procedure LAND IMMEDIATELY TOTAL ELECTRICAL POWER LOSS DUE TO FAULTY BATTERY CONNECTION All Electrical Equipment EXCEPT MASTER – OFF ALT RESTART – PRESS AND HOLD 5 SECONDS Electrical Equipment - ON |  |  |
| --- | --- | --- | --- |