DISCLAIMER – MAJOR PORTIONS OF THIS DOCUMENT HAVE BEEN RETYPED. IT IS HERE TO FAMIARIZE YOU WITH THE DATA IN THIS AD. REFER TO OFFICIAL SOURCES WHEN COMPLYING WITH THIS AD.

TO MINIMIZE THE POSSIBLE ADVERSE AFFECTS OF UNDETECTED REAR ENGINE STOPPAGE DURING TAKE-OFF

71-17-08

CESSNA

Amendment 39-1273

Applicability:  Applies to Model 337 Airplanes.

Compliance: To minimize the possible adverse affects of undetected rear engine stoppage During take-off, within the next 10 hours' time in service after the effective date of this AD, the following modifications must be accomplished and the following operational

procedures will be applicable:

A) Readjust the rear engine idle RPM from 575-625 to 625-675 and readjust idle mixture accordingly.

B) Taxi primarily by use of the rear engine.

C) Initiate all take-offs by advancing the throttle on the rear engine to a point where the normal functioning of the rear engine has been established before advancing the throttle of the front engine.

D) Install a permanent placard to the right of the tachometer instrument to read as follows:

TAXI AND TAKE-OFF

LEAD WITH REAR ENGINE POWER

CHECK RPM AND FUEL FLOW

NOTE

The operator may make and install a temporary placard using minimum 1/8 inch high letters until the permanent placard is obtained from the manufacturer and properly installed.

Cessna Service Letter ME 71-21 refers to the subject matter of this AD.

This amendment becomes effective August 24, 1971.