DISCLAIMER – MAJOR PORTIONS OF THIS DOCUMENT HAVE BEEN RETYPED. IT IS HERE TO FAMIARIZE YOU WITH THE DATA IN THIS AD. REFER TO OFFICIAL SOURCES WHEN COMPLYING WITH THIS AD.

TO DETECT LEAKAGE OF FLAMMABLE FLUIDS FROM FLEXIBLE HOSE ASSEMBLIES IN THE ENGINE COMPARTMENT

71-24-04

CESSNA

Amendment 39-1340

Applicability:  Applies to following airplanes:

MODELS              	SERIAL NUMBERS AFFECTED

177                 		661, 17700001 and 17700003 thru 17701164

206                 		206-0001 thru 206-0275

U206                	U206-0276 thru U206-0437

P206                	P206-0002 thru P206-0160

U206A               	U206-0438 thru U206-0656

P206A               	P206-0161 thru P206-0306

TU206A              	U206-0487 thru U206-0656

P206B               	P206-0307 thru P206-0419

TU206B              	U206-0657 thru U206-0914

TP206A              	P206-191 thru P206-0306

TP206B              	P206-0307 thru P206-0419

U206B               	U206-0657 thru U206-0914

P206C, TP206C	P206-0420 thru P206-0519

U206C, TU206C	U206-0915 thru U206-1234

210D                	21058221 thru 21058510

210E                	21058511 thru 21058715

T210F               	T210-0001 thru T210-0197

210F                	21058716 thru 21058818

T210G               	T210-0198 thru T210-0307

210G                	21058819 thru 21058936

T210H               	T210-0307 thru T210-0392

210H                	21058937 thru 21059062

336                 		633, 636, 336-001 thru 336-0195

337                 		337-0002 thru 337-0239

337A                	337-0240 thru 337-0525 (except 337-0306 and 337-0470)

337B                	337-0001, 337-0470, 337-0526 thru 337-0755

T337B               	337-0001, 337-0526 thru 337-0755 (except 337-0569)

337C, T337C         	337-0756 thru 337-0978

Compliance: Required as indicated, unless already accomplished.

To detect leakage of flammable fluids from flexible hose assemblies in the engine compartment within 100 hours' time in service after the effective date of this AD and thereafter at intervals not to exceed 100 hours' time in service, accomplish the following:

A. Visually, or by any other method approved by FAA, inspect fuel lines as follows:

1. Pressurize the fuel lines with boost pump operating in high position.  When accomplishing this test, the mixture control should be in the idle cutoff position.

2. With fuel lines pressurized, examine all flexible hose exteriors in the engine compartment for evidence of fuel stains, wetness, or leakage.

3. After pressure testing fuel hoses, allow sufficient time for excess fuel to drain overboard from the engine manifold before attempting an engine start.

B. Visually, or by any other method approved by the FAA, inspect oil lines for evidence of wetness or leakage.

C. If, as a result of the inspections required by Paragraphs A or B, fuel and/or oil stain, wetness or leaking is found, replace with a serviceable hose assembly.

D. Inspections required by Paragraphs A and B will no longer be required when hose assemblies are replaced with assemblies having a Cessna dated metal tag attached.  TSO-C53a Type C approved hose assemblies are also acceptable for this replacement. (See Note 1)

NOTE

11: This does not preclude continued inspections of this area as required by FAR 91.

NOTE

Cessna Service Letter SE71-7, Supplement No. 1 dated November 3, 1971, covers this subject.

The above airplanes use Aeroquip 601 series or Stratoflex 156 series hose assemblies available under Cessna P/N S1236-x-xxxx. The hose assemblies are identified by a steel braided exterior.

This amendment becomes effective November 23, 1971.