DISCLAIMER – MAJOR PORTIONS OF THIS DOCUMENT HAVE BEEN RETYPED. IT IS HERE TO FAMIARIZE YOU WITH THE DATA IN THIS AD. REFER TO OFFICIAL SOURCES WHEN COMPLYING WITH THIS AD.

TO PREVENT UNWANTED PROPELLER ROTATION

77-12-08

CESSNA

Amendment 39-2918

Applicability:  Applies to the following models and serial numbers of airplanes equipped with external electrical ground power

receptacles:

MODELS		SERIAL NUMBERS

172M			17265685 thru 17267584

172N			17267585 thru 17268080

R172K		R1722000 thru R1722109

177B	17702314 thru 17702374, 17702376 thru 17702395, 17702397, 17702398, 17702400, 17702401, 17702403, 17702404, 17702405, and 17702407 thru 17702409

177RG		177RG0788 thru 177RG0822, 177RG0824 and 177RG0826

180J			18052621 thru 18052770

180K			18052771 thru 18052777

182P			18264296 thru 18265175

182Q			18265176 thru 18265220

A185F			18502839 thru 18503165

U206F/TU206F	U20603021 thru U20603521

U206G/TU206G	U20603522 thru U20603547

207/T207		20700315 thru 20700362

207A/T207A		20700363 thru 20700365

210L/T210L		21061040 thru 21061573

337G			33701672 thru 33701748

T337G		P3370226 thru P3370257

Compliance: Required as indicated, unless already accomplished.

To prevent unwanted propeller rotation, at the next use of the external ground power receptacle, but not later than September 15, 1977, accomplish the following:

A) Test the aircraft's electrical starter system by applying power to the external electrical ground power receptacle.  If the propeller does not rotate, make a maintenance record entry indicating that this AD has been accomplished.

If the propeller rotates, modify the electrical system in accordance with Cessna Service Letters SE76-11, dated May 10, 1976, SE77-1, dated January 17, 1977, and SE77-1 (Supplement #1), dated May 2, 1977, or later approved revisions, as applicable.

NOTE

When performing this test take adequate precautions to assure that no person or object will be struck by the propeller if rotation occurs.

B) Any equivalent method of compliance with this AD must be approved by the Chief, Engineering and Manufacturing Branch, FAA, Central Region.

This amendment becomes effective June 27, 1977.