DISCLAIMER – MAJOR PORTIONS OF THIS DOCUMENT HAVE BEEN RETYPED. IT IS HERE TO FAMIARIZE YOU WITH THE DATA IN THIS AD. REFER TO OFFICIAL SOURCES WHEN COMPLYING WITH THIS AD.

TO DETECT CRACKING OF THE WING FRONT AND REAR SPAR LOWER CAPS, FRONT SPAR WEB AND WEB DOUBLER

78-09-05

CESSNA

Amendment 39-3202

Applicability:  Applies to the following models and serial number airplanes certificated in all categories:

Model				Serial  Numbers

336					336-0001 through 336-0195

337, 337A, 337B, T337B,		337-0001 through 337-1193

337C, T337C, 337D and T337D

337E, T337E, 337F, T337F,		33701194 through 33701852

337G, 337H and T337H

T337G and P337H			P3370001 through P3370313

M337B				All serial numbers

Compliance: Required as indicated in accordance with Compliance Table I set forth in this AD or as otherwise set forth herein, unless already accomplished.

To detect cracking of the wing front and rear spar lower caps, front spar web and web doubler - In

accordance with instructions set forth herein and in Cessna Multi-Engine Service Letter ME78-2,

dated February 13, 1978, or later revisions:

I. At time intervals noted in Table I of this AD, inspect the right and left wing front and rear spar

lower cap and front spar web and web doubler on airplanes having model and serial numbers

shown in Table 1.

TABLE  I - COMPLIANCE TIMES

__________________________________________________________________________________________________________________

Airplane Type/		Total Time in			Inspection Times for Spar			Inspection Times for

Operation		Service for each		Cap-Inspections required			Front Spar Web - Inspections

of the following:			by Paragraph I.A.				Required by Paragraph I.B.

1) Front Spar			Initial              		Interval for		Initial			Interval for

Lower Cap			Inspection		Repetitive		Inspection		Repetitive

2) Rear Spar						Inspections					Inspections *

Lower Cap

3) Front Spar

Web & Web

Doubler

_____________________________________________________________________________________________________________

Non-Pressurized	0 to 4999			None			None			None			None

(See Note 1)

5,000 & up			Within 25		500 hours		Within 25		500 hours

hours time in					hours time

service after                       			in service

the effective                        			after the

date of this AD                      			effective

for those air-                      			date of this

planes which                         			AD.

have not yet

been inspected

in accordance

with AD 76-10-11

or 73-04-02 or;

within 500 hours

time-in-service

after the last

inspection in

accordance with

AD 76-10-11 or

73-04-02.

TABLE  I COMPLIANCE TIMES

__________________________________________________________________________________________________________________

Airplane Type/		Total Time in			Inspection Times for Spar			Inspection Times for

Operation		Service for each		Cap-Inspections required			Front Spar Web - Inspections

of the following:			by Paragraph I.A.				Required by Paragraph I.B.

Initial              		Repetitive		Initial			Repetitive

Pressurized          	0 to 9,999           		None                  	None          		None            		None

10,000 & up          		Within 25 hours       	500 hours      		Within 25       		500 hours

time in service                      			hours time-

after the effective                  			in-service

date of this AD                      			after the

for those airplanes                  			effective date

which have not yet                   			of this AD

been inspected

in accordance with

AD 76-10-11 or

73-04-02 or;

within 500 hours

time-in-service

after the last

inspections in

accordance with

AD 76-10-11 or

73-04-02.

NOTE 1:

# For those airplanes  	0 to 2,999           		None                  	None           		None             		None

which have engaged   	3,000 & up           		Within 25 hours       	300 hours     		Within 35        		300 hours

in contour or                             				time-in-service					hours time-in-

terrain following                         				after the effective                  			service after    		*After initial

operations at low                         			date of this AD                      			the effective    		inspection in

altitudes, such                            				for those air-                       			date of this    		accordance

as power/pipeline                         			planes which have                    			AD.              		With this AD

patrol, fish or                           				not yet been                                          					the compliance

game spotting,                            				inspected in                                          					time for repetitive

aerial applications,                      				accordance with the                                   				inspections may

police patrol, live-                      				suggestion in                                         					be adjusted to

stock management,                         			AD 76-10-11 or                                        					allow compliance

etc. Cessna                               				73-04-02 or, with-                                    					at the same time

recommends and FAA                        			in 300 hours time-                                    				as the inspections

strongly urges                            				in-service after                                      					required by

inspections at                            				the last inspection                                   					paragraph 1A of

intervals shown to                        			in accordance with                                    				this AD.

the right of this                         				AD 76-10-11 or                                        					note.                                     		  .							73-04-02                                              							.

##### A. Front and rear spar lower cap inspection

Model                                 			Serial Numbers	        .

336                                   			336-0001 through 336-0195

337, 337A, 337B, T337B, 337C,  		337-0001 through 337-1193

T337C, 337D, and T337D

337E, T337E, 337F, T337F, and 337G    	33701194 through 33701548

T337G                                 			P3370001 through P3370138

M337B                                 			All serial numbers

1. Front spar lower cap inspection.

a. Inspect three fastener holes on the left wing and three fastener holes on the right wing for wing spar cracks using eddy current inspection procedures outlined in the above noted Cessna Service Letter.  Figure 1 shows the area involved and the three fasteners (two NAS 221 screws at W.S. 64.41 and the jack point bolt hole at W.S. 68.45) that are to be inspected.

b. Remove the two NAS 221 screws at Wing Station 64.41 one at a time for this inspection and hold the boom fairing firmly against underlying thicknesses of material to insure proper eddy current probe depth settings during this inspection.  A cross-section with the screws at W.S. 64.41 removed is shown in Figure 2.  Figure 3 shows the spar assembly (less boom and wing skins) and the relationship of the lower cap to the other parts at the strut attachment.  Figure 4 shows the spar cross-section thru the jack point bolt hole.

c. If cracks are found in either the right or left wing front spar lower cap during any inspection required by this AD, prior to approving the airplane for return to service replace the front spar lower cap in both the right and left wing with new spar caps.

2. Rear spar lower cap inspection.

a. Dye penetrant inspect the lower spar cap area between two and three inches (2nd rivet outboard of W.S. 66.00 rib) outboard of wing station 66.00 rib for spar cap cracks originating in the rivet hole in accordance with inspection provisions in the above noted Cessna Service Letter.  Figures 5,  6  and  7  show the location of the area on the rear spar lower cap to be inspected.

b. If cracks are found in either the right or left wing rear spar lower cap during any inspection required by this AD, prior to approving the airplane for return to service replace the rear spar lower cap in both the right and left wing.

##### B. Front spar web and web doubler inspection

Model                                    			Serial Numbers

336                                      			336-0001 through 336-0195

337, 337A, 337B, T337B, 337C,            	337-0001 through 337-1193

T337C, 337D and T337D

337E, T337E, 337F, T337F, 337G,          	33701194 through 33701852

337H and T337H

T337G and P337H                          		P3370001 through P3370313

M337B                                    		All serial numbers

1. Remove wing root access panels and wing root fairings.

2. Visually inspect the radii of both the spar web and web doubler for cracks in the

shaded critical area shown in Figure 8 of this AD.

3. If cracks are found in either the right or left wing front spar web or web doubler during any inspection required by this AD, prior to approving the airplane for return to service, replace the discrepant components.

II. Airplanes found to have cracked spar caps, webs or web doublers during inspections required by this AD may be flown in accordance with FAR 21.197 to a base where the component replacement can be accomplished.

III. Accomplish the repetitive inspections made mandatory by this AD on those spar caps, webs and web doublers replaced in accordance with this AD upon accumulation of the total times-in-service shown in Table I of this AD.  Repetitive inspections "strongly urged" via Note 1 in Table I of this AD should also be accomplished on spar caps, webs and web doublers replaced in accordance with this AD upon accumulation of the total times-in-service shown in Note 1, Table I, of this AD.

IV. Notify in writing the Chief, Engineering and Manufacturing Branch, FAA, Central Region, of  the location and length of any crack found during inspections required by this AD and also the total time in service of the component at the time the crack was discovered.  (Reporting approved by the Office of Management and Budget under OMB NO. 04-R0174.)

V. The time interval for repetitive inspections required by this AD, after compliance with initial inspection requirements, can be adjusted up to 25 hours to allow accomplishment of these inspections at regular scheduled maintenance periods.

VI. Equivalent methods of compliance with this AD must be approved by the Chief, Engineering and Manufacturing Branch, FAA, Central Region.

This AD supersedes AD 76-10-11, Amendment 39-2621 (41 FR 22405, 22406, 22407 and 22408).

This Amendment becomes effective May 11, 1978.

## FIGURE 1 - WING STRUT ATTACHMENT AREA – WITH STRUT CUFF REMOVED

### Wing Station 64.41

#### FIGURE 2 - LOWER SPAR CAP AT FASTENERS

#### FIGURE 3 - FRONT WING SPAR-BOOM & STRUT ATTACHMENT AREA

#### FIGURE 4 - LOWER SPAR CAP AT JACK POINT

FIGURE 5 - AREAS FOR INSPECTION OF LOWER CAPS OR REAR SPARS

FIGURE 6 – TANK COVER REMOVED FROM TOP OF WING

FIGURE 7 - AREA ON LOWER REAR SPAR CAP TO BE INSPECTED

WS

23.00              	     UPPER SPAR CAP

#### FIGURE 8 - FRONT SPAR ASSEMBLY